# RoadSense: Speed Safety Score for Safer Roads

ADB AI for Safer Roads Innovation Challenge — Findings Summary
Team: Neuron Nexus | June 2026

# 1. Executive Summary

RoadSense analyzes 14,793 road segments across Thailand and Maharashtra using GPS probe data, road network geometry, and street-level imagery to produce a Speed Safety Score (0–1) for every segment. The score measures how well posted speed limits align with the Safe System approach — the principle that human bodies can only withstand impact speeds of 30–50 km/h depending on road context.

Key finding: 77.9% of road segments have mean operating speeds above the Safe System threshold for their road type and land use. 12.2% of network length (11,445 km) is classified Critical — requiring immediate intervention. Secondary urban roads in both countries are the most dangerous category, with mean operating gaps exceeding 20 km/h above survivable impact speeds.

# 2. Methodology Overview

RoadSense implements two independent scoring approaches for cross-validation.

## 2.1 4-Component Composite Score

Four weighted factors per segment:

| Component | Weight | What it measures |
| --- | --- | --- |
| Limit Misalignment | 30% | How far the posted limit exceeds Safe System recommendations |
| Operating Speed Risk | 30% | How far actual traffic (85th percentile speed) exceeds Safe System thresholds |
| VRU Exposure | 25% | Pedestrian fatality risk × road class / land use exposure |
| Traffic Volume | 15% | Normalised traffic volume percentile |

## 2.2 3-Module RoadSense Score

Three modules at weights 35%, 35%, 30%: Module A — Speed Alignment (average of limit gap and operating speed gap), Module B — VRU Exposure (land use, road class, speed risk, and PTW context), and Module C — Road Environment (infrastructure gap proxy plus speed mismatch penalty). Both approaches produce scores in [0, 1].

## 2.3 Risk Tier Classification

| Risk Tier | Score Range | Action Required |
| --- | --- | --- |
| Critical — Immediate Review | ≥ 0.65 | Immediate Safe System intervention |
| High — Priority Review | 0.45 – 0.65 | Priority review and countermeasure design |
| Moderate — Scheduled Review | 0.25 – 0.45 | Scheduled review and monitoring |
| Low — Monitor | < 0.25 | Maintain current approach |

# 3. Key Findings

## 3.1 Network-Level Risk Distribution

| Risk Tier | Length (km) | % of Network |
| --- | --- | --- |
| Critical — Immediate Review | 11,445 | 12.2% |
| High — Priority Review | 19,313 | 20.7% |
| Moderate — Scheduled Review | 25,499 | 27.3% |
| Low — Monitor | 37,253 | 39.8% |

Mean network score: 0.415 (moderate risk)

## 3.2 Critical Findings

Speed limits are systematically too high. 77.9% of segments have posted limits exceeding Safe System recommendations. Average limit misalignment: 13 km/h on urban primary roads.

Traffic routinely exceeds survivable speeds. On 76.6% of segments, 85th percentile operating speeds exceed Safe System thresholds. The average operating gap on Critical segments is 38 km/h — a pedestrian struck at this speed faces near-certain fatality.

Secondary urban roads are the most dangerous category. These roads (mixed-use with foot traffic and shops) have a Safe System limit of 30 km/h but carry posted limits of 50–60 km/h and actual speeds above 60 km/h.

VRU exposure amplifies risk. 35% of segments have high VRU exposure scores (>0.6), concentrated in urban areas near schools and markets.

## 3.3 Regional Findings

| Metric | Thailand | Maharashtra |
| --- | --- | --- |
| Total segments analyzed | 9,732 | 5,061 |
| Mean Speed Safety Score | 0.428 | 0.395 |

# 4. Policy Recommendations

## 4.1 Immediate Priorities (Critical Tier — 11,445 km)

Audit posted speed limits on all Critical segments. Many carry limits 20–40 km/h above Safe System recommendations. Lowering limits is the fastest, lowest-cost intervention.

Deploy traffic calming on secondary urban roads with operating speeds >50 km/h. Speed humps, raised crossings, and lane narrowing reduce speeds by 10–15 km/h.

Prioritize pedestrian crossings at VRU-critical segments near schools and markets (marked crossings, pedestrian refuges, signalised crossings).

## 4.2 Medium-Term Actions (High Tier — 19,313 km)

Reclassify road hierarchies where the posted limit and road function are mismatched.

Strengthen enforcement with automated speed cameras where operating speeds exceed posted limits by >15 km/h.

Integrate VRU risk into road design standards — high-VRU segments should trigger protected pedestrian infrastructure in any redesign.

## 4.3 Systemic Changes

Adopt Safe System speed limits as the national default (e.g., all urban secondary roads default to 30 km/h).

Build a data-driven speed management program — run the RoadSense pipeline quarterly to track changes and evaluate interventions.

Expand data collection: pedestrian and cyclist counts at VRU-critical locations, street-level imagery collection, and crash data integration.

# 5. Replication in Data-Scarce Countries

The RoadSense methodology is designed for replication across Asia-Pacific. Key adaptations:

| Data Source | If Unavailable | Alternative |
| --- | --- | --- |
| GPS probe data | Many LMICs lack probe data | Use OSM road classification + design speed as proxy |
| Mapillary imagery | Limited coverage | CLIP zero-shot classification on available street photos |
| Official speed limits | Not digitised | Extract from OSM maxspeed tags (~40% coverage in most countries) |
| POIs (schools, markets) | Always available via OSM | Overpass API provides global coverage at no cost |
| Traffic volume data | No volume data | Remove Volume component; redistribute weight to other factors |

Minimal viable deployment requires only: OSM road network, OSM speed limit tags, OSM points of interest, and any available speed data (even 50 spot speed measurements can calibrate the model).

# 6. Validation & Limitations

## Validation

Internal consistency: Both scoring approaches produce correlated rankings (r > 0.85), confirming robustness to weighting choices.

Spatial autocorrelation: Moran's I confirms high-risk segments cluster spatially (p < 0.01), validating the score captures genuine geographic patterns.

Sensitivity analysis: Varying component weights by ±10% preserves >95% of tier classifications.

## Limitations

No crash data validation. The score measures risk factors (speed, exposure, infrastructure proxies) rather than observed crashes — a deliberate Safe System design choice.

Imagery coverage gaps. Module C relies on infrastructure proxies rather than actual infrastructure detection. Full Mapillary/CLIP integration would improve accuracy.

Sample size bias. Segments with <500 observations are excluded, potentially biasing toward higher-volume roads.

Single time period. Seasonal and temporal speed/volume variations are not captured.

# References

World Health Organization. (2021). Global Plan for the Decade of Action for Road Safety 2021–2030. Geneva: WHO.

ITF/OECD. (2018). Speed and Crash Risk. International Transport Forum Research Report. Paris: OECD Publishing.

Asian Development Bank. (2026). AI for Safer Roads 2026 — Speed and Volume Data for Thailand and Maharashtra.

OpenStreetMap contributors. (2026). Planet dump. https://planet.openstreetmap.org